SolGames game board assembling *

| Item side |  |  | Solder side |  |  |
| --- | --- | --- | --- | --- | --- |
|  | + 12 В | 1А | 1В | +12 В |  |
|  | GND | 2А | 2В | GND |  |
|  |  | 3А | 3В | GND |  |
|  | HOLD 1 SW | 4А | 4В | Reserve |  |
|  | HOLD 2 SW | 5А | 5В | Reserve |  |
|  | HOLD 3 SW | 6А | 6В | Reserve |  |
|  | HOLD 4 SW | 7А | 7В | Reserve |  |
|  | HOLD 5 SW | 8А | 8В | Reserve |  |
|  | START SW | 9А | 9В | Reserve |  |
|  | BET/RISK SW | 10А | 10В | Reserve |  |
|  | Reserve | 11А | 11В | Reserve |  |
|  | Reserve | 12А | 12В | Reserve |  |
|  | Reserve | 13А | 13В | Reserve |  |
|  | MAX BET SW | 14А | 14В | Reserve |  |
|  | HELP SW | 15А | 15В | Reserve |  |
|  | FRONT DOOR SW | 16А | 16В | BACKDOOR SW |  |
|  | TOTAL IN | 17А | 17В | TOTAL OUT |  |
|  | COIN A | 18А | 18В | COIN B |  |
|  | Reserve | 19А | 19В | Reserve |  |
|  | STATISTIC SW | 20А | 20В | SERVICE SW |  |
|  | Reserve | 21А | 21В | KEY OUT SW |  |
|  | Reserve | 22А | 22В | Reserve |  |
|  | BILL COUNTER | 23А | 23В | Reserve |  |
|  | Reserve | 24А | 24В | Reserve |  |
|  | Reserve | 25А | 25В | Reserve |  |
|  | Reserve | 26А | 26В | Reserve |  |
|  | Reserve | 27А | 27В | Reserve |  |
|  | BILL ACCEPTOR LOCK | 28А | 28В | Reserve |  |
|  | HOLD 1 LAMP | 29А | 29В | BET/RISK LAMP |  |
|  | HOLD 2 LAMP | 30А | 30В | MAXBET LAMP |  |
|  | HOLD 3 LAMP | 31А | 31В | Reserve |  |
|  | HOLD 4 LAMP | 32А | 32В | Reserve |  |
|  | HOLD 5 LAMP | 33А | 33В | Reserve |  |
|  | START LAMP | 34А | 34В | HELP LAMP |  |
|  | Reserve | 35А | 35В | Reserve |  |
|  | GND | 36А | 36В | GND |  |

SolGames boards are connected only to the power supply of АТХ standard through the standard plug and socket unit. Other slot machine units (buttons, keys, BILL ACCEPTOR) must be connected to the same power supply.